Nobel Systems, Inc

405-843-8835

NEC SV-8100

Multiline Telephone User Guide

All Digital and

IP Phones

With Voice Mail

NEC SV-8100 - User Introduction for Display Phone

To adjust the HANDSET volume………………………………………………….…….. 5

To adjust LCD contrast: …………………………………………………………….……… 5

To adjust Speakerphone volume: …………………………………………….………. 5

To adjust your phones ringer: …………………………………………………….…….. 5

Display Phone Buttons

Phone Buttons………………………………………………………….…. 6

SOFTKEYS Explained

Your phone’s display while in the idle mode………………………….……….. 6

Softkey display when “DN (down arrow)” is pressed…………………….……..7

Softkey display when “LIST” is pressed……………………………………….……..7

Softkey display when Redial is pressed……………………………………………..7

Softkey display when CID is pressed…………………………………….…….……..8

Softkey display when “DIR” is pressed…………………………………….…….…..8

Your phone’s display when the handset is lifted or the

speaker button is pressed prior to making a all…………………....………..9

Your phone’s display during an external call………………………………….…..9

Using Phone

PLACING CALLS - Internal Calls…………………………………………..10

PLACING CALLS - Outside Calls………………………………………..…10

### Changing how you receive intercom and

### transferred calls……………………………………………………………10

### MICROPHONE CONTROL……………………………………………….....10

### BACKGROUND MUSIC……………………………………………………..10

### TRANSFERRING CALLS…………………………………………………….10

### HOLDING CALLS……………………………………………………………11

CALL FORWARDING………………………………………………………..11

Call Forwarding Off-Premise (Both internal and external)

SPEAKERPHONE CALLS……………………………………………………12

GROUP LISTENING………………………………………………………….12

CONFERENCE……………………………………………………………….13

CALLER ID………………………………………………………………..…14

CALL PARK – SYSTEM ……………………………………………………..15

TRUNK QUEUING …………………………………………………..….……15

LAST # REDIAL ……………………………………………………………..16

CALL PICKUP ……………………………………………………………….16

DIRECTED CALL PICKUP……………………………………………………16

EXTERNAL GROUP PAGING…………………………………………………17

DO NOT DISTURB (DND) ……………………………………………………18

STATION SPEED DIAL – PROGRAMMING………………………………….18

DIALING SYSTEM/STATION SPEED DIALS…………………………………19

PROGRAMMING ONE-TOUCH ICM KEYS

ONE-TOUCH ICM KEYS……………………………………………………………………..18

## LINE KEYS – PROGRAMMABLE LINE KEYS

Function Number - Function Additional Data……………………………………20

Program your extension name……………………………………………21

Feature Codes…………………………………………………………………………….………..22

Voice Mail Initial Set Up

Initial Set Up…………………………………………………………………………………..23

# Using Voice Mail

Logging into Your Mailbox………………………………………….……23

Listen to your messages……………………………………………………… ….……..24

Record a greeting for your mailbox……………………………………….…….…..24

Record a name for your mailbox ……………………………………………….…...24

Enter a security code for your mailbox ……………………………………….…..24

Transfer a call directly to a mailbox ……………………………………………….24

Record a quick message (from outside)………………………………………….25

forward all incoming calls to your mailbox when you

will be out of the office for a extended time……………………………………25

Inmail Voice Mail System Admin Guide

Delete a Security code……………………………………………………………….….…26

Change the Main Greeting…………………………………………………………..……26

Speed Dial……………………………………………………………………………….………26

Change Time…………………………………………………………………………….……..27

Change how intercom calls ring your extension………………………….…….27

Change the pitch of your incoming ring (multiline terminal only)………27

Adjust the volume of your ring………………………………………………………….27

NEC SV-8100 - User Introduction for Display Phone

Non-display and 6 button w/o joystick

6/12/24 button phones w/joystick

THE JOYSTICK

RAISESE

LOWERSE

To adjust the HANDSET volume:

Press RAISE or LOWER buttons on the joystick (or the UP and DOWN pointing arrows) with the handset off-hook.

To adjust LCD contrast:

Press RAISE or LOWER buttons on the joystick (or the UP and DOWN pointing arrows) with the handset on-hook.

To adjust Speakerphone volume:

Press RAISE or LOWER buttons on the joystick (or the UP and DOWN pointing arrows) with the speaker button ON.

To adjust your phones ringer:

Press SPEAKER and dial 729. While the phone is ringing press the RAISE or LOWER buttons on the joystick (or the UP and DOWN pointing arrows) with the speaker button ON.

Display Phone Buttons

HOLD. The red HOLD button at the bottom of the phone is used to put calls on hold.

TRANSFER. The TRANSFER button is used to transfer calls to another phone.

SPEAKER. The SPEAKER button activates the speakerphone.

ANSWER. The ANSWER button is used to answer a second call, like call waiting.

RECALL. The RECALL button is used to transfer calls if you have Centrex or Centranet lines only. Otherwise this button can be used to disconnect a call and give you fresh dial tone.

FEATURE. The FEATURE button is used for programming.

MIC. The MIC button turns on/off the built in microphone. Most prefer to leave the

microphone on (light on) so you can use the speakerphone and talk back intercom.

SOFT KEYS. The 4 gray buttons just below the LCD screen are called soft keys. These buttons can be used for a variety of things depending upon what you are doing on the phone.  The Exit button terminates any current softkey function and returns to the normal display. The HELP button does not function.

## SOFTKEYS Explained

21 OCT THU 12:34PM

101	           	        STA101

List      Dir      VMsg       DN

Your phone’s display while in the idle mode.

LIST – Opens softkey display containing [REDIAL & CID]

DIR – Opens the directory [SPD, EXT., STA, TELBK]

VMsg – takes you to YOUR voice mailbox

DN(a downward pointing arrow) – Next screen with more options.

STA101 – Is replaced with your name

21 OCT THU 12:34PM

ICM     PROG                   UP

Softkey display when “DN (down arrow)” is pressed

ICM – Opens ICM Menu screen (InPg, ExPg, P/U)

PROG – Press this key to program your 10-station speed dials.

UP (upward pointing arrow) – Returns to Main screen

----------------------------------------------------------------------------------------------------------------

21 OCT THU 12:34PM

Redial    CID

Softkey display when “LIST” is pressed

Redial– Pressing this key brings up the Redial screen.

CID – Pressing this key brings up the Caller ID screen.

----------------------------------------------------------------------------------------------------------------

21 OCT THU 12:34PM

UP       DN        Delete     Save

Softkey display when Redial is pressed.

UP/DN (upward & downward pointing arrows) – Allows scrolling through the last 10 numbers.

NOTE: See Redial section in this guide for more detailed instructions.

21 OCT THU 12:34PM

2174423800     DIGITALCOMM

UP       DN        Delete     Save

Softkey display when CID is pressed.

UP/DN (upward & downward pointing arrows) – used to scroll through the last 20 numbers that rang or were transferred to your phone (included VM calls).

NOTE: See Caller ID section in this guide for more detailed instructions.

----------------------------------------------------------------------------------------------------------------

21 OCT THU 12:34PM

SPD     EXT.     STA     TELBK

Softkey display when “DIR” is pressed

SPD – Press this key to open the SYSTEM speed dial screen for accessing 1000 system numbers.

EXT. – Press this key to open a screen listing the extension numbers by name.

STA – Press this key to open your personal bank of 10 station numbers.

TELBK – Press this key to open the programmed telephone book.

21 OCT THU 12:34PM

InPg      ExPg     P/U

Your phone’s display when the handset is lifted or the speaker button is pressed prior to making a call.

InPg – Press this key to page an internal intercom group.

ExPg – Press this key to page thru an external paging system.

P/U – Press this key to use group or directed call pickup.

----------------------------------------------------------------------------------------------------------------

21 OCT THU 12:34PM

CONF    RPT      SAVE    TRF

Your phone’s display during an external call.

CONF – Press this key to begin a conference call.

RPT –

SAVE – Press this key to save the currently dialed number for later retrieval.

TRF – Press this key to transfer this call (functions like the TRANSFER button.

## PLACING CALLS - Internal Calls

1. Lift the handset.

2. Dial a station number or 0 for the attendant. OR Press the button programmed for Direct Station Select.

3. Voice announce after the tone burst or wait for the ringing call to be answered.

Note 1: When calling a multiline telephone, dialing 1 after the station number will change ringing to voice or voice to ringing.

Note 2: To directly access a personal voice mailbox, dial 8 after dialing the station number.

6

### PLACING CALLS - Outside Calls

1. Lift the handset.

2. Typically dial 9 OR press an idle Outside Line key.

3. Dial the telephone number.

### Changing how you receive intercom and transferred calls

1. Lift the handset.

2. Dial 721 for handsfree answer back (display will show VOICE) or 723 to make your phone ring (display will show RINGING).

3. Hang up.

## MICROPHONE CONTROL

Press the MIC key.

A lit MIC LED indicates that the MIC is on.

## BACKGROUND MUSIC

To turn Background Music on or off:

1. Press the Speaker key.

2. Dial 725.

3. Press Speaker to hang up.

## TRANSFERRING CALLS

With a call in progress:

1. Press the Transfer key.

2. Dial the station number.

3. Announce the call (optional).

4. Replace the handset.

Note 1: If the called station is busy, replace the handset to initiate a camp-on. Unanswered camp-ons and unscreened transfers will recall to the transferring station.

Note 2: To return to the original party, press the GREEN flashing key.

Call Transfer using programmed DSS keys

With a call in progress:

1. Press the programmed DSS button.

3. Announce the call (optional).

4. Replace the handset.

Note 1: A Programmable Function Key may be assigned for DSS. (See programming a function in this manual)

Note 2: To transfer a call directly to a personal voice mailbox, dial 8 after dialing the station number.

## HOLDING CALLS

To retrieve a call you placed on hold using the HOLD button, press the GREEN flashing key.

Note: Unless your system is programmed with a key for each outside line, calls placed on hold using the Hold button can ONLY BE PICKED UP AT YOUR extension.

CALL FORWARDING

All Call Forwarding (Both internal and external)

To set call forward all:

Press the Speaker key

Dial 741

Dial 1 to Set

Dial the extension you wish to have your calls answered by or the VM Pilot Number  ______

Hang up

To cancel when you return, dial 741, then press 0 to cancel

NOTE: If you call forward all your calls to another station and that station is in turn call forwarded (either busy/no answer or all calls), you calls will end up in YOUR mailbox.

Call Forwarding Off-Premise (Both internal and external)

To set call forward all:

Press the Speaker key

Dial 741

Dial 1 to Set

Dial 9 + the phone number you wish your calls forwarded to. Hang up

To cancel when you return, dial 741, then press 0 to cancel

NOTE: If you have to dial 1+the area code to reach this number make sure to include them after the 9)

SPEAKERPHONE CALLS

1. Press the Speaker key and the LED lights.

2. Ensure that the MIC LED is lit.

3. Place internal or outside call.

4. Press the Speaker key to disconnect call.

Note: The handset may be used at any time during the conversation. To resume handsfree operation or to monitor a call, press the Speaker key (the LED lights) and replace the handset.

## GROUP LISTENING

To initiate Group Listening:

1. Place or answer call using the handset.

2. Press Speaker twice (but do not hang up).

The Speaker button flashes slowly.

21 OCT THU 12:34PM

GROUP LISTEN

Conf

Note 1: You can talk to the caller through your handset. Your co-workers hear your caller’s voice over your telephone’s speaker.

Note 2: Pressing Speaker once turns your Speakerphone on; second press turns on Group Listening; third press cancels Group Listening.

## CONFERENCE

1. Establish first intercom or trunk call. (display shows).

21 OCT THU 12:34PM

CONF    RPT      SAVE    TRF

2. Press Conf softkey (display shows).

21 OCT THU 12:34PM

AnHd    Dir

3. Dial the extension you want to add. - OR -

Access outside call by dialing 9 - OR – press “DIR” to access the directory. (AnHd – returns you to call 1)

4. When second party answers (display shows), press Add to add this call or Rls to end second call.

21 OCT THU 12:34PM

Add      Rls

5. After Add has been pressed display shows.

21 OCT THU 12:34PM

Begin    Add

6. When you are ready to being the conference press Begin

softkey. Press Add to continue adding to the conference call.

NOTE: Unless a conference bridge has been installed, A conference call can consist of three parties. Two parties within your office and one outside caller or Two outside callers and one inside extension.

CALLER ID

Answer

Receive incoming ringing or transferred outside call:

1. Review the telephone display for the calling party’s name or number.

2. Answer the call accordingly.

Temporary Memory

An unanswered call will cause the Call History key to flash (a handset icon in the upper left hand corner of the display), indicating a new call has been placed in the temporary memory.

Press the center button of the joystick. The display will change to:

[1] MISSED CALLS

[2] VOICE MAIL

Dial 1 for Missed calls.

21 OCT THU 12:34PM

2174423800     DIGITALCOMM

UP       DN        Delete     Save

Use up and down arrows to scroll through the log.

To place a call back to a number in the temporary memory list, with the number to be dialed displayed, lift the handset or press the Speaker key.

21 OCT THU 12:34PM

2174423800     DIGITALCOMM

One       All

To delete, press Delete key and follow the softkeys.

## CALL PARK - SYSTEM

To Park a call in a system orbit:

Note: You can Park Intercom or trunk calls.

1. With a call in progress - press a Park key (Park 1, Park 2, etc).

Note: The Park key LED lights.

2. Use Paging to announce call.

3. Press the hookswitch to disconnect quietly.

Note: If not picked up, the call will recall to you.

- OR -

1. With a call in progress - press the Transfer key.

2. Dial #6 and the Park orbit (01~64).

Note: If you hear busy tone, the orbit is busy. Try another orbit.

3. Use Paging to announce call.

4. Press the hookswitch to disconnect quietly.

Note: If not picked up, the call will recall to you.

To pick up a parked call:

1. Lift the handset.

2. Press the red blinking Park key (Park 1, Park 2, etc).

- OR -

1. Press the Speaker key.

2. Dial *6 and the Park orbit (01~64).

## TRUNK QUEUING

(Use this feature when all lines are busy)

1. Lift the handset and dial 9.

2. When a busy signal is heard (the display will say BUSY)

Press the # button and hang up.

When a trunk becomes free - your extension will ring.

Pick up the handset and hear OUTSIDE dial tone.

Dial your number without dialing the access code 9.

NOTE: If you do not answer your phone within 10 seconds, the queuing will be automatically cancelled.

## LAST # REDIAL

1. Without lifting the handset, press the Redial on the joystick. The last dialed number is displayed.

2. To redial the last number press #. OR Search for the desired number from the Redial List by pressing the Redial Softkey or VOLUME UP or VOLUME DOWN keys.

3. Lift the handset or press Speaker to place the call.

## CALL PICKUP

Group Call Pickup

21 OCT THU 12:34PM

#### InPg      ExPg     P/U       DN

To answer a call ringing another telephone in your Pickup Group:

1. Pick up the handset or press the Speaker key.

21 OCT THU 12:34PM

#### P/U       Page       Grp

2. Press the P/U key

3. Press the GRP key to answer the call.

## DIRECTED CALL PICKUP

To use Directed Call Pickup to intercept a call to a co-worker’s extension:

1. Pick up the handset or press the Speaker key.

2. Dial **.

3. Dial the number of extension whose call you want to intercept.

Note: If more than one call is coming in, the system sets the priority for which calls it will answer first.

## INTERNAL GROUP PAGING

To make an Internal Page announcement:

1. Lift the handset.

21 OCT THU 12:34PM

#### InPg      ExPg     P/U       DN

2. Press “InPg” – and enter the internal paging group number (1-9) at the next screen.

- OR -

1. Lift the handset and dial 701 and the Paging Zone number (0~9).

Note: Dialing 0 calls All Call Internal Paging.

2. Dial *1 and the Combined Paging Group code 1~8 or 0 (for Internal/External All Call).

3. Make announcement.

4. Press the hookswitch to disconnect quietly, then hang up.

Note: Display indicates the Combined Paging as an External Page.

## EXTERNAL GROUP PAGING

To Page an external zone:

21 OCT THU 12:34PM

#### InPg      ExPg     P/U       DN

1. Lift the handset.

2. Press “ExPg” – and enter the internal paging group number at the next screen.

- OR -

1. Lift the handset and dial 703 and the Paging Zone number (0~9).

2. External Paging Group code (1~9 or 0 for Internal/External All Call).

3. Make announcement.

4. Press the hookswitch to disconnect quietly, then hang up.

Note: Display indicates the Combined Paging as an External Page.

Note: If the Internal Page Zone is busy or if there are no extensions in a page group, the page may be announced as an

External Page only.

## DO NOT DISTURB (DND)

To set DND

1. Press pre-programmed DND feature button.

2. Dial 3 to set DND All Calls – the display will indicate

“DND ALL”

To cancel DND

1. Press pre-programmed DND feature button.

2. Dial 0 – the display will indicate

“DND CANCEL”

## PROGRAMMING ONE-TOUCH ICM KEYS

1. Press the Speaker key.

2. Dial Service Code 751.

3. Press the Line Key to be programmed. Screen should say “Not Defined” or an old entry you can erase!  Leave CAP Keys in place or you will not be able to transfer calls!

4. Dial 01 followed by the extension number.

5. Press HOLD then Speaker

## STATION SPEED DIAL - PROGRAMMING

To store a Station Speed Dialing number (display telephones only):

1. Press the Speaker key.

2. Dial 755.

3. Dial the Station Speed Dial buffer number to be programmed (0~9).

1 = Station Speed Dial buffer 1

2 = Station Speed Dial buffer 2 etc.

Note: 0 = Station Speed Dial buffer 10

4. Dial the Access Code (e.g., 9) - if required.

5. Dial telephone number you want to store (up to 24 digits).

Note: Valid entries are 0~9, # and *. To enter a pause, press MIC. To store a Flash, press Recall.

6. Press Hold.

7. Enter the name associated with the Speed Dialing number (display telephones only):

# # = Accepts an entry (only required if two letters on the same key are needed - ex: TOM). Pressing # again = Space.  Conf Clears the character entry one character at a time (used when entering the Name). EXIT Clears all the entries from the point of the flashing cursor and to the right (used when entering the Number, this clears both the number and name).

DIALING SYSTEM/STATION SPEED DIALS

21 OCT THU 12:34PM

101	           	        STA101

List      Dir      VMsg       DN

To dial a Speed dialing number:

1. Press the DIR key.

21 OCT THU 12:34PM

SPD     EXT.     STA     TELBK

2. Press the SPD key for SYSTEM numbers or STA key for STATION numbers.

3. The [A] in the upper right corner number designates the letter A as the alphabetical starting point. To look for name starting with an N, press 6 twice.

DIRECTORY DIALING         [A]

SYS: COMMON

UP       DN        DIAL            STA     TELBK

Use the up arrow to scroll all speed

numbers/names starting at N and proceeding through all programmed speed dial numbers. When you find the number you wish to call – press CALL – the speakerphone will activate and the number will be dialed.

Note: This procedure is the same for both system and station speed dialing. “SYS: COMMON” is replaced with “STATION” for station speed numbers.

This space left blank for your notes:

## LINE KEYS – PROGRAMMABLE LINE KEYS

The following functions below can be assigned to Line Keys if allowed by Class of Service.

To Program a Line key as a Programmable Function Key:

1. Press the Speaker key.

2. Dial Service Code 751.

3. Press the Line Key to be programmed.

4. Dial the code for the desired feature plus additional data if required.

5. Press Speaker to hang up.

Note: Service Code 00 will erase the function from the key.

Note: In some cases, the Hold key needs to be pressed before pressing the Speaker key.

CAUTION: CHECK WITH YOUR DIGITAL COMMUNICATIONS TECHNICIAN BEFORE CHANGING ANY FEATURE KEYS.  YOU COULD MAKE YOUR PHONE INOPERATABLE.

Function Number - Function Additional Data

01 DSS / One-Touch Extension number or any numbers (up to 24 digits). Press HOLD to write.

03 DND Key

04 BGM (ON/OFF)

05 Headset

08 Incoming Call Log

09 Day/Night Mode Switch Mode number (1~8)

10 Call Forward - Immediate

11 Call Forward - Busy

12 Call Forward - No Answer

13 Call Forward - Busy/No Answer

14 Call Forward - Both Ring

15 Follow Me

19 External Group Paging External Paging Number (1~8)

20 External All Call Paging

21 Internal Group Paging Internal Paging Number (01~64)

22 Internal All Call Paging

23 Meet-Me Answer to Internal Paging

24 Call Pickup

25 Call Pickup for Another Group

26 Call Pickup for Specified Group /Call Pickup Group Number

27 Speed Dial -Common/ Private Speed Dial Number (Common/ Private)

28 Speed Dial - Group Speed Dial number (Group)

Program your extension name:

1. Press the Speaker key

2. Dial 700

3. Press the Hold key if you are changing your ext or for another ext dial

that ext number

4. Enter the new name

5. Press the Hold key

6. Press the speaker key to hang up.

Note: The name will not change on that ext until the handset is lifted up

after the change has been made.

### Feature Codes

Night Mode Switching(Attendant Feature only)--------------718

Setting System Time (Attendant Feature only)----------------728

Call Forward All----------------------------------------------------741

Call Forward Busy-------------------------------------------------742

Call Forward No Answer---------------------------------------743

Call Forward B/NA------------------------------------------------744

Alarm Clock-------------------------------------------------------------721

Background Music-------------------------------------------------725

Do Not Disturb-------------------------------------------------------747

Extension Name Change---------------------------------------700

Answer Message Waiting-------------------------------------*0

Cancel All Messages Waiting--------------------------------773

Cancel Message Waiting---------------------------------------771

Program Function Key-------------------------------------------751

(2-digit Service code)

Program Function Key-------------------------------------------752

(3 Digit Service Codes)

Change Ring Tones------------------------------------------------720

Check Ring Tones---------------------------------------------------711

Ring Volume Set-----------------------------------------------------729

Station Speed Dial Set-------------------------------------------755

Headset Mode Switching---------------------------------------688

Headset Ring Volume Set--------------------------------------662

Internal/External All Page-------------------------------------*10

Door Box Access-----------------------------------------------------702

Station Speed Dialing---------------------------------------------#7

VM Access (InMail & VMS)-------------------------------------*8

Voice Mail Initial Set Up

Users with Display Phones can use the softkeys at the top of the phone as well.

Access Voice Mail by pressing the softkey under VMsg

Use the soft keys at the top of the phone to maneuver quicker Press softkey under VMsg

When asked to enter a security code, enter the default of 0000.  Be sure to change it.

21 OCT THU 12:34PM

101	           	        STA101

List      Dir      VMsg       DN

A tutorial will then start.  Follow along and complete all tasks.  DO NOT HANG UP UNTIL THE VOICE MAIL LADY SAYS ”Great”

# Using Voice Mail

Your new Voice Mail System is operated using the 4 softkeys under your display.
Your Mailbox Number is the same as your telephone extension number.

Start by Logging into Your Mailbox

There are three ways to login in to your mailbox to retrieve your messages.

1. To login local:

- Press "VMsg" softkey

When asked to enter a security code, enter the default of 0000.  Be sure to change it.

2. To login remote thru auto attendant: (outside office}

Dial your company’s main number or dial ______________________

When auto attendant answers press #

System will prompt for mail box number.

Enter your personal station or mail box number.

When asked to enter a security code, enter the default of 0000.  Be sure to change it.

.

3. To login remote thru live attendant: (outside office}

Dial your company’s main number

When attendant answers ask to be transferred to station 700

Voice mail will answer

press #

System will prompt for mail box number.

Enter your personal station or mail box number.

When asked to enter a security code, enter the default of 0000.  Be sure to change it.

To listen to your messages:
1. Push Lstn softkey:
2. While listening to your message, you can do one of the following: 
- Dial RE (73) to record a reply for the message sender.
- Dial MF (63) to forward the message to a co-worker.
- Dial MC (62) to make a return call to the message sender.
- Dial TI (84) to hear the time and date the message was sent.
- Dial SA (72) to save the message in your mailbox.
- Push Erase to erase the message.
- Push Next to listen to the next message.
- Dial B (2) to back the message up a few seconds, or G (4) to go ahead a few seconds.
- Push Rpt to repeat the message from the beginning.
- Dial * to pause and resume listening.
- Push Exit to exit the message listen mode.

To record a greeting for your mailbox:
1. Push Greet softkey.
2. Push Gr1, Gr2 or Gr3 to select which of the three available greetings you want to be active.
3. Do one of the following:
- Push Lstn to listen to your greeting (if recorded).
- Push Rec to record a new greeting.
- Push Erase to delete your greeting (and use the built in prerecorded greeting).
- Push Exit to exit without changing your greeting.

To record a name for your mailbox:
1. Dial RN (76).
2. Do one of the following:
- Push Lstn to listen to your name (if recorded).
- Push Rec to record a new name.
- Push Erase to erase your name (and use your extension number instead). 
- Push Exit to exit without changing your name.

To enter a security code for your mailbox:
2. Dial OP (67) and push CODE.
3. Do one of the following:
- Enter a new security and push OK.
- Push Erase to erase your security code.
- Push Exit to exit without changing your security code.

To transfer a call directly to a mailbox:

Press Transfer

Dial the extension number +8

Hang up.

To record a quick message (from outside)

Dial the Auto Attendant number (typically after hours)

Wait for main greeting to start, then dial *, followed by the mailbox number

Leave message and hang up.

To forward all incoming calls to your mailbox when you will be out of the office for a extended time.

Press the Speaker key

Dial 741

Dial 1 to Set

Dial the VM Pilot Number  ______

Hang up

To cancel when you return, dial 741, and then press 0 to cancel

This space left blank for your notes:

# Inmail Voice Mail System Admin Guide

To Delete a Security code, the greeting, messages or recorded name in a

personal voicemail box:

1. From any phone with Admin rights (Usually ext 101)

2. Press the message button

3. Enter security code if there is one

4. Dial 72 to access System Admin

5. Press SUBS softkey (in display)

6. Dial the extension number you want to change

7. Press the available softkey in the display or 7 to delete the security

code.

To Change the Main Greeting:

1. From any phone with Admin rights (Usually ext 101)

2. Press the Message Button

3. Enter the Security Code

4. Dial 72 (admin Options)

You are now in System Administrator Menu

5. In the display press INSTR

6. When asked for the mailbox number. Dial 001 for Day Greeting or 002 for

Night Greeting

7. In the display press REC to record your new greeting

Speed Dials:

1. Press Speaker

2. Dial 753 (system) 754 (group) or 755 (station)

3. Dial storage code number 1 – 0 for station, 000 through 999 for system

4. Dial the access code (i.e. 9) if required

5. Dial telephone number you want to store (up to 24 digits)

6. Press Hold

7. Enter the name associated with the speed dialing number

8. Press Hold

9. Press the speaker key to hang up.

Note: To enter a pause, pres MIC. To store a Flash, press Recall.

# = space. You will need this if you need a letter that is on the

same number key.

## will put a space between names

Conf = Delete last space

Exit – Clears all the entries from the point of the flashing cursor

and to the right.

Change Time:

1. Press the Speaker key

2. Dial 728

3. Dial two digits for the hour (24 hour clock, 13 = 1:00 PM)

4. Dial two digits for the minutes (00~59)

5. Press Speaker to hang up

To change how intercom calls ring your extension:

1. Press the speaker key or lift the handset

2. Dial 723 to have calls ring your extension.

-OR-

3. Dial 721 to have calls voice announce to your extension.

To change the pitch of your incoming ring (multiline terminal only):

1. Press the Speaker key

2. Dial 720

3. Dial 1 to change ringing for Intercom calls

-ORDial

2 to change ringing for trunk calls.

4. Select the Pitch (1~8)

5. Press Speaker to hang up.

To adjust the volume of your ring:

1. Press the Speaker key

2. Dial 729

3. Press the Up and Down arrow keys to adjust the volume

This space left blank for your notes:

# For more help call Nobel Systems, Inc 405-843-8835 local, 800-575-8835 toll free or visit our website @    www.nobelvoicedata.com